The design, construction, and maintenance of stormwater management facilities are governed by the specifications included in Section 9.4 of the Henrico County Environmental Compliance Manual.  The following checklist only identifies the information and details that must be included in the SWM plan.

VA DEQ DESIGN SPEC. NO. 1:  ROOFTOP DISCONNECTION

(mark the appropriate BMP that is being reviewed)

Simple Rooftop Disconnection

Rooftop Disconnection with:

Soil compost-amended filter path (also refer to VA DEQ Design Spec #4 – Soil Compost Amendment)

Micro-infiltration practice (also refer to VA DEQ Design Spec #8 - Infiltration Practices)

Micro-bioretention practice (also refer to attached VA DEQ Design Spec #9 - Bioretention)

Rainwater harvesting (also refer to attached VA DEQ Design Spec #6 – Rainwater Harvesting)

Urban bioretention (also refer to attached VA DEQ Design Spec #9, Appendix 9-A – Urban Bioretention)

MINIMUM DESIGN CRITERIA:

- All Simple Rooftop Disconnections must meet the following minimum design criteria.  The items checked below have not been properly addressed:

The impervious (rooftop) area treated must be 1,000 sq. ft. or less per discharge point;

The longest flow path (roof/gutter) must be 75 feet or less;

The disconnection length must be equal to the longest flow path (and at least 40 feet);

The disconnection slope must be < 2% (< 5% with the use of EC-2 or EC-3);

Downspouts must be extended min. 5 feet from buildings or foundations if grade <1%;

External pre-treatment is required and must be specified on the plans (leaf screens, etc.).

- All Rooftop Disconnections with a soil compost-amended filter path must meet the following minimum design criteria.  The items checked below have not been properly addressed:

The impervious (rooftop) area treated must be 1,000 sq. ft. or less per discharge point;

The longest flow path (roof/gutter) must be 75 feet or less;

Flow from the downspout must be spread over a 10-foot wide strip;

The filter path must be at least 20 feet;

A flow spreading device must be installed at the downspout;

The filter path must be depressed and level to contain the downspout flow without concentrating flow to the center of the filter path;

2 to 4 inches of compost is required and must be tilled to 6-10 inches within the filter path.

Downspouts must be extended min. 5 feet from buildings or foundations if grade <1%;

External pre-treatment is required and must be specified on the plans (leaf screens, etc.).

- All Rooftop Disconnections with micro-infiltration must meet the following minimum design criteria.  The items checked below have not been properly addressed:

The impervious (rooftop) area treated must be no more than 2,500 sq. ft. or less per discharge point;

The maximum micro-infiltration depth is 3 feet;

The minimum allowable native soil infiltration rate is 0.5 inches per hour;

One soils test is required per micro-infiltration unit;

Downspouts must be extended min. 5 feet from buildings or foundations if grade <1%;

The micro-infiltration must be located at least 5 feet from the building (down gradient) or 25 feet from the building (up gradient);

External pre-treatment is required and must be specified on the plans (leaf screens, etc.);

There must be at least 2’ between bottom of the infiltration unit and the seasonal high water table.

- All Rooftop Disconnections with micro-bioretention must meet the following minimum design criteria.  The items checked below have not been properly addressed:

The impervious (rooftop) area treated must be 1,000 sq. ft. or less per discharge point;

For Level 1 micro-bioretention, the surface area of the micro-bioretention must be at least 5% of the contributing rooftop drainage area;

For Level 1 micro-bioretention, an underdrain and gravel layer must be provided;

For Level 1 micro-bioretention, the media depth must be at least 18 inches;

For Level 2 micro-bioretention, the surface area of the micro-bioretention must be at least 6% of the contributing rooftop drainage area;

For Level 2 micro-bioretention, the media depth must be at least 24 inches;

The minimum allowable native soil infiltration rate is 0.5 inches per hour or an underdrain system must be provided;

The bioretention corridor must have a minimum effective length of at least 20 feet;

One soils test is required per micro-bioretention unit;

The micro-bioretention unit must be located at least 5 feet from the building (down gradient) or 25 feet from the building (up gradient);

Downspouts must be extended min. 5 feet from buildings or foundations if grade <1%;

External pre-treatment is required and must be specified on the plans (leaf screens, etc.).

- All Rooftop Disconnections with rainwater harvesting must meet the following minimum design criteria.  The items checked below have not been properly addressed:

Also refer to attached checklist for VA DEQ Design Spec #6 – Rainwater Harvesting;

Downspouts must be extended min. 5 feet from buildings or foundations if grade <1%;

External pre-treatment is required and must be specified on the plans (leaf screens, etc.).

- All Rooftop Disconnections with urban bioretention must meet the following minimum design criteria.  The items checked below have not been properly addressed:

Also refer to attached checklist for VA DEQ Design Spec #9, Appendix 9-A – Urban Bioretention

Downspouts must be extended min. 5 feet from buildings or foundations if grade <1%;

External pre-treatment is required and must be specified on the plans (leaf screens, etc.).

ADDITIONAL COMMENTS:

PLAN REQUIREMENTS:

- Provide the locations of rooftop disconnection and the layout and dimensions of the alternative practices (in plan, profile, and section views), if applicable, including:

The longest flow path;

The rooftop drainage area to each discharge point;

The disconnection length;

The disconnection slope;

The distance downspouts are extended from buildings or foundations;

Any pre-treatment specifications and/or call-outs;

The type of inflow to secondary practice (sheet flow or roof leader);

The alternative practices, relative to the rooftop disconnection, if applicable.

- Provide construction notes that address the BMP installation in the sequence of construction.

- The plans must clearly indicate that the SWM facility has been designed in accordance with Section 9.4.1 (Rooftop Disconnect) of the Henrico County Environmental Compliance Manual.

ADDITIONAL COMMENTS:

MAINTENANCE:

- A Maintenance Agreement is required, identifying the person or organization responsible for maintenance and authorizing access for inspections and maintenance.

- Provide a summary of the long term maintenance requirements for the SWM facility on the SWM plan.

- Inspections required by the Maintenance Agreement must be conducted by 1) a person who is licensed as a professional engineer, architect, landscape architect, or land surveyor pursuant to Sec. 54.1-400 et seq. of the Code of Virginia; 2) a person who works under the direction and oversight of a licensed professional engineer, architect, landscape architect, or land surveyor; or 3) a person who holds a Stormwater Inspector or Stormwater Combined Administrator certificate of competence from the State Water Control Board.

ADDITIONAL COMMENTS:

CONSTRUCTION:

- The sequence of construction must address the SWM facility installation and appropriate inspections, including:  initial site preparation, excavation/grading, and installation of the embankment, principal outlet structure, and emergency spillway.  We recommend the County staff be involved in these inspections.

- The sequence of construction must clearly state that a construction record drawing and certification that the stormwater management facility has been constructed in accordance with the approved plan must be submitted to the County and approved prior to Environmental Compliance Bond (ECB) release.

ADDITIONAL COMMENTS:

Reviewed by:

Date Reviewed:

Phone Number:

July 2014